/*!

\page chamfer_operation_page Chamfer

\n To produce a \b Chamfer in the <b>Main Menu</b> select

<b>Operations - > Chamfer</b>

\n This operation allows you to make chamfer of the edges of a Shape.

\n The \b Result will be a \b GEOM_Object.

\n To create chamfer on all edges of the given shape, you need to

define the <b>Main Object</b> to create a chamfer on and the \b

Dimension (radius) of the chamfer.

\n <b>TUI Command:</b> <em>geompy.MakeChamferAll(Shape, D)</em>

\n <b>Arguments:</b> Name + 1 SHAPE + 1 value (Chamfer dimension).

\n <b>Advanced options</b> \ref preview_anchor "Preview"

\image html chamfer1.png

\image html chamfer_all.png "Chamfer on the whole object"

\n In the algorithms below there exist two ways to define the \b

Dimension of the Chamfer

\n Firstly the \b Dimension can be defined via \b D1 and \b D2, which

represent the offset on the first and the second face forming the

edge.

\n Secondly the \b Dimension can be defined via \b D, which represents

the offset on the first face, and the \b Angle, which is the angle

between the first face and the chamfer plane.

\n To create chamfer on the specified edges of the given shape,  you

need to define the <b>Main Object</b> to create a fillet on, select

the two faces to which belongs the necessary common edge in the viewer

and define the \b Dimension of the Chamfer.

\n <b>TUI Command:</b> <em>geompy.MakeChamferEdge(Shape, D1, D2,

Face1, Face2),</em> where \em Shape is a shape to create a chamfer on,

\em D1 is a chamfer size along \em Face1, \em D2 is a chamfer size

along \em Face2, \em Face1 and \em Face2 are indices of faces in

Shape.

\n <b>Alternative TUI Command:</b> <em>geompy.MakeChamferEdge(Shape, D, Angle,

Face1, Face2),</em> where \em D1 is a chamfer size along \em Face1 and

\em Angle is the angle between Face1 and the chamfer plane.

\image html chamfer2.png

\image html chamfer_edge.png "Chamfer on the specified edges"

\n To create chamfer on the specified faces of the given shape,  you need

to define the <b>Main Object</b> to create a fillet on, select the

necessary faces the OCC Viewer and define the \b Dimension of the

Chamfer.

\n <b>TUI Command:</b> <em>geompy.MakeChamferFaces(Shape, D1, D2,

ListOfFaceID),</em> where \em Shape is a shape to create chamfer on,

\em D1 and D2 are chamfer sizes along faces from  \em

ListOfFaceID. For each Edge Face1 and Face2 are defined basing

on their indices.

\n <b>Alternative TUI Command:</b> <em>geompy.MakeChamferFaces(Shape, D, Angle,

ListOfFaceID),</em> where \em D1 is a chamfer size along \em Face1 and

\em Angle is the angle between Face1 and the chamfer plane.

\image html chamfer3.png

\image html chamfer_faces.png "Chamfer on the specified faces"

\n To create chamfer on an arbitrary set of edges of the given shape

to define the <b>Main Object</b> to create a fillet on, select the

necessary edges the OCC Viewer and define the \b Dimension of the

Chamfer.

\n <b>TUI Command:</b> <em>geompy.MakeChamferEdges(Shape, D1, D2,

ListOfEdgeID),</em> where \em Shape is a shape to create chamfer on,

\em D1 and D2 are chamfer sizes along faces defined by the algorithm

basing on \em ListOfEdgeID, which is a list of indices of edges in

\em Shape. For each Edge the algorithm selects Face1 and Face2 basing

on their indices.

\n <b>Alternative TUI Command:</b> <em>geompy.MakeChamferEdges(Shape, D, Angle,

ListOfEdgeID),</em> where \em D1 is a chamfer size along \em Face1 and

\em Angle is the angle between Face1 and the chamfer plane.

\image html chamfer4.png

Our <b>TUI Scripts</b> provide you with useful examples of the use of

\ref tui_chamfer "Transformation Operations".

*/